6809 REFERENCE CARD

-------------------

-----------------------------------------------------------------------------------------------------------------------------

|Instruction| Forms | Addressing Modes                                              | Description       | Condition Codes   |

|           |       |---------------------------------------------------------------|                   |-------------------|

|           |       | Immediate || Direct    || Indexed(1)|| Extended  || Inherent  |                   | b5| b3| b2| b1| b0|

|           |       |-----------||-----------||-----------||-----------||-----------|                   |---|---|---|---|---|

|           |       | Op| ~ | # || Op| ~ | # || Op| ~ | # || Op| ~ | # || Op| ~ | # |                   | H | N | Z | V | C |

|===========|=======|===|===|===||===========||===|===|===||===|===|===||===|===|===|===================|===|===|===|===|===|

| ABX       |       |   |   |   ||   |   |   ||   |   |   ||   |   |   || 3A| 3 | 1 | B+X ->X (Unsigned)| . | . | . | . | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| ADC       | ADCA  | 89| 2 | 2 || 99| 4 | 2 || A9| 4+| 2+|| B9| 5 | 3 ||   |   |   | A+M+C ->A         | * | * | * | * | * |

|           | ADCB  | C9| 2 | 2 || D9| 4 | 2 || E9| 4+| 2+|| F9| 5 | 3 ||   |   |   | B+M+C ->A         | * | * | * | * | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| ADD       | ADDA  | 8B| 2 | 2 || 9B| 4 | 2 || AB| 4+| 2+|| BB| 5 | 3 ||   |   |   | A+M ->A           | * | * | * | * | * |

|           | ADDB  | CB| 2 | 2 || DB| 4 | 2 || EB| 4+| 2+|| FB| 5 | 3 ||   |   |   | B+M ->B           | * | * | * | * | * |

|           | ADDD  | C3| 4 | 3 || D3| 6 | 2 || E3| 6+| 2+|| F3| 7 | 3 ||   |   |   | D+M:M+1 ->D       | . | * | * | * | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| AND       | ANDA  | 84| 2 | 2 || 94| 4 | 2 || A4| 4+| 2+|| B4| 5 | 3 ||   |   |   | A^M ->A           | . | * | * | 0 | . |

|           | ANDB  | C4| 2 | 2 || D4| 4 | 2 || E4| 4+| 2+|| F4| 5 | 3 ||   |   |   | B^M ->B           | . | * | * | 0 | . |

|           | ANDCC | 1C| 3 | 2 ||   |   |   ||   |   |   ||   |   |   ||   |   |   | CC^IMM ->CC       |   |   |   |   |(6)|

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| ASL       | ASLA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 48| 2 | 1 | A                 |(7)| * | * | * | * |

|           | ASLB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 58| 2 | 1 | B ||<-|||||||||<-0|(7)| * | * | * | * |

|           | ASL   |   |   |   || 08| 6 | 2 || 68| 6+| 2+|| 78| 7 | 3 ||   |   |   | M  c  b7 <<< b0   |(7)| * | * | * | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| ASR       | ASRA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 47| 2 | 1 | A                c|(7)| * | * | . | * |

|           | ASRB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 57| 2 | 1 | B b7>|||||||||->|||(7)| * | * | . | * |

|           | ASR   |   |   |   || 07| 6 | 2 || 67| 6+| 2+|| 77| 7 | 3 ||   |   |   | M    b7 >>> b0    |(7)| * | * | . | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| BIT       | BITA  | 85| 2 | 2 || 95| 4 | 2 || A5| 4+| 2+|| B5| 5 | 3 ||   |   |   | Bit Test A (M^A)  | . | * | * | 0 | . |

|           | BITB  | C5| 2 | 2 || D5| 4 | 2 || E5| 4+| 2+|| F5| 5 | 3 ||   |   |   | Bit Test B (M^B)  | . | * | * | 0 | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| CLR       | CLRA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 4F| 2 | 1 | 0 ->A             | . | 0 | * | 0 | 0 |

|           | CLRB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 5F| 2 | 1 | 0 ->B             | . | 0 | * | 0 | 0 |

|           | CLR   |   |   |   || 0F| 6 | 2 || 6F| 6+| 2+|| 7F| 7 | 3 ||   |   |   | 0 ->M             | . | 0 | * | 0 | 0 |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| CMP       | CMPA  | 81| 2 | 2 || 91| 4 | 2 || A1| 4+| 2+|| B1| 5 | 3 ||   |   |   | Compare M from A  |(7)| * | * | * | * |

|           | CMPB  | C1| 2 | 2 || D1| 4 | 2 || E1| 4+| 2+|| F1| 5 | 3 ||   |   |   | Compare M from B  |(7)| * | * | * | * |

|           | CMPD  | 10| 5 | 4 || 10| 7 | 3 || 10| 7+| 3+|| 10| 8 | 4 ||   |   |   | Comp. M:M+1 from D| . | * | * | * | * |

|           |       | 83|   |   || 93|   |   || A3|   |   || B3|   |   ||   |   |   |                   |   |   |   |   |   |

|           | CMPS  | 11| 5 | 4 || 11| 7 | 3 || 11| 7+| 3+|| 11| 8 | 4 ||   |   |   | Comp. M:M+1 from S| . | * | * | * | * |

|           |       | 8C|   |   || 9C|   |   || AC|   |   || BC|   |   ||   |   |   |                   |   |   |   |   |   |

|           | CMPU  | 11| 5 | 4 || 11| 7 | 3 || 11| 7+| 3+|| 11| 8 | 4 ||   |   |   | Comp. M:M+1 from U| . | * | * | * | * |

|           |       | 83|   |   || 93|   |   || A3|   |   || B3|   |   ||   |   |   |                   |   |   |   |   |   |

|           | CMPX  | 8C| 4 | 3 || 9C| 6 | 2 || AC| 6+| 2+|| BC| 7 | 3 ||   |   |   | Comp. M:M+1 from X| . | * | * | * | * |

|           | CMPY  | 10| 5 | 4 || 10| 7 | 3 || 10| 7+| 3+|| 10| 8 | 4 ||   |   |   | Comp. M:M+1 from Y| . | * | * | * | * |

|           |       | 8C|   |   || 9C|   |   || AC|   |   || BC|   |   ||   |   |   |                   |   |   |   |   |   |

-----------------------------------------------------------------------------------------------------------------------------

-----------------------------------------------------------------------------------------------------------------------------

|Instruction| Forms | Addressing Modes                                              | Description       | Condition Codes   |

|           |       |---------------------------------------------------------------|                   |-------------------|

|           |       | Immediate || Direct    || Indexed(1)|| Extended  || Inherent  |                   | b5| b3| b2| b1| b0|

|           |       |-----------||-----------||-----------||-----------||-----------|                   |---|---|---|---|---|

|           |       | Op| ~ | # || Op| ~ | # || Op| ~ | # || Op| ~ | # || Op| ~ | # |                   | H | N | Z | V | C |

|===========|=======|===|===|===||===========||===|===|===||===|===|===||===|===|===|===================|===|===|===|===|===|

| COM       | COMA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 43| 2 | 1 | /A ->A            | . | * | * | 0 | 1 |

|           | COMB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 53| 2 | 1 | /B ->B            | . | * | * | 0 | 1 |

|           | COM   |   |   |   || 03| 6 | 2 || 63| 6+| 2+|| 73| 7 | 3 ||   |   |   | /M ->M            | . | * | * | 0 | 1 |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| CWAI      |       | 3C|20+| 2 ||   |   |   ||   |   |   ||   |   |   ||   |   |   | CC^IMM ->CC(&Wait)|   |   |   |   |(6)|

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| DAA       |       |   |   |   ||   |   |   ||   |   |   ||   |   |   || 19| 2 | 1 | Decimal Adjust A  | . | * | * | 0 | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| DEC       | DECA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 4A| 2 | 1 | A -1 ->A          | . | * | * | * | . |

|           | DECB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 5A| 2 | 1 | B -1 ->B          | . | * | * | * | . |

|           | DEC   |   |   |   || 0A| 6 | 2 || 6A| 6+| 2+|| 7A| 7 | 3 ||   |   |   | M -1 ->M          | . | * | * | * | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| EOR       | EORA  | 88| 2 | 2 || 98| 4 | 2 || A8| 4+| 2+|| B8| 5 | 3 ||   |   |   | A xor M ->A       | . | * | * | 0 | . |

|           | EORB  | C8| 2 | 2 || D8| 4 | 2 || E8| 4+| 2+|| F8| 5 | 3 ||   |   |   | B xor B ->B       | . | * | * | 0 | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| EXG       | R1,R2 | 1E| 8 | 2 ||   |   |   ||   |   |   ||   |   |   ||   |   |   | R1 <-> R2(2)      | . | . | . | . | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| INC       | INCA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 4C| 2 | 1 | A+1 ->A           | . | * | * | * | . |

|           | INCB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 5C| 2 | 1 | B+1 ->B           | . | * | * | * | . |

|           | INC   |   |   |   || 0C| 6 | 2 || 6C| 6+| 2+|| 7C| 7 | 3 ||   |   |   | M+1 ->M           | . | * | * | * | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| JMP       |       |   |   |   || 0E| 3 | 2 || 6E| 3+| 2+|| 7E| 4 | 3 ||   |   |   | EA(3) ->PC        | . | . | . | . | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| JSR       |       |   |   |   || 9B| 7 | 2 || AD| 7+| 2+|| BD| 8 | 3 ||   |   |   | Call Subroutine   | . | . | . | . | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| LD        | LDA   | 86| 2 | 2 || 96| 4 | 2 || A6| 4+| 2+|| B6| 5 | 3 ||   |   |   | M ->A             | . | * | * | 0 | . |

|           | LDB   | C6| 2 | 2 || D6| 4 | 2 || E6| 4+| 2+|| F6| 5 | 3 ||   |   |   | M ->B             | . | * | * | 0 | . |

|           | LDD   | CC| 3 | 3 || DC| 5 | 2 || EC| 5+| 2+|| FC| 6 | 3 ||   |   |   | M:M+1 ->D         | . | * | * | 0 | . |

|           | LDS   | 10| 4 | 4 || 10| 6 | 3 || 10| 6+| 3+|| 10| 7 | 4 ||   |   |   | M:M+1 ->S         | . | * | * | 0 | . |

|           |       | CE|   |   || DE|   |   || EE|   |   || FE|   |   ||   |   |   |                   |   |   |   |   |   |

|           | LDU   | CE| 3 | 3 || DE| 5 | 2 || EE| 5+| 2+|| FE| 6 | 3 ||   |   |   | M:M+1 ->U         | . | * | * | 0 | . |

|           | LDX   | 8E| 3 | 3 || 9E| 5 | 2 || AE| 5+| 2+|| BE| 6 | 3 ||   |   |   | M:M+1 ->X         | . | * | * | 0 | . |

|           | LDY   | 10| 4 | 4 || 10| 6 | 3 || 10| 6+| 3+|| 10| 7 | 4 ||   |   |   | M:M+1 ->Y         | . | * | * | 0 | . |

|           |       | 8E|   |   || 9E|   |   || AE|   |   || BE|   |   ||   |   |   |                   |   |   |   |   |   |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| LEA       | LEAS  |   |   |   ||   |   |   || 32| 4+| 2+||   |   |   ||   |   |   | EA(3) ->S         | . | . | . | . | . |

|           | LEAU  |   |   |   ||   |   |   || 33| 4+| 2+||   |   |   ||   |   |   | EA(3) ->U         | . | . | . | . | . |

|           | LEAX  |   |   |   ||   |   |   || 30| 4+| 2+||   |   |   ||   |   |   | EA(3) ->X         | . | . | * | . | . |

|           | LEAY  |   |   |   ||   |   |   || 31| 4+| 2+||   |   |   ||   |   |   | EA(3) ->Y         | . | . | * | . | . |

-----------------------------------------------------------------------------------------------------------------------------

-----------------------------------------------------------------------------------------------------------------------------

|Instruction| Forms | Addressing Modes                                              | Description       | Condition Codes   |

|           |       |---------------------------------------------------------------|                   |-------------------|

|           |       | Immediate || Direct    || Indexed(1)|| Extended  || Inherent  |                   | b5| b3| b2| b1| b0|

|           |       |-----------||-----------||-----------||-----------||-----------|                   |---|---|---|---|---|

|           |       | Op| ~ | # || Op| ~ | # || Op| ~ | # || Op| ~ | # || Op| ~ | # |                   | H | N | Z | V | C |

|===========|=======|===|===|===||===========||===|===|===||===|===|===||===|===|===|===================|===|===|===|===|===|

| LSL       | LSLA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 48| 2 | 1 | A                 |(7)| * | * | * | * |

|           | LSLB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 58| 2 | 1 | B ||<-|||||||||<-0|(7)| * | * | * | * |

|           | LSL   |   |   |   || 08| 6 | 2 || 68| 6+| 2+|| 78| 7 | 3 ||   |   |   | M  c  b7 <<< b0   |(7)| * | * | * | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| LSR       | LSRA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 44| 2 | 1 | A                 | . | 0 | * | . | * |

|           | LSRB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 54| 2 | 1 | B 0->|||||||||->||| . | 0 | * | . | * |

|           | LSR   |   |   |   || 04| 6 | 2 || 64| 6+| 2+|| 74| 7 | 3 ||   |   |   | M    b7 >>> b0   c| . | 0 | * | . | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| MUL       |       |   |   |   ||   |   |   ||   |   |   ||   |   |   || 3D|11 | 1 | AxB ->D (Unsigned)| . | . | * | . |(8)|

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| NEG       | NEGA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 40| 2 | 1 |/A+1 ->A           |(7)| * | * | * | * |

|           | NEGB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 50| 2 | 1 |/B+1 ->B           |(7)| * | * | * | * |

|           | NEG   |   |   |   || 00| 6 | 2 || 60| 6+| 2+|| 70| 7 | 3 ||   |   |   |/M+1 ->M           |(7)| * | * | * | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| NOP       |       |   |   |   ||   |   |   ||   |   |   ||   |   |   || 12| 2 | 1 | No operation      | . | . | . | . | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| OR        | ORA   | 8A| 2 | 2 || 9A| 4 | 2 || AA| 4+| 2+|| BA| 5 | 3 ||   |   |   | A or M ->A        | . | * | * | 0 | . |

|           | ORB   | CA| 2 | 2 || DA| 4 | 2 || EA| 4+| 2+|| FA| 3 | 3 ||   |   |   | B or M ->B        | . | * | * | 0 | . |

|           | ORCC  | 1A| 3 | 2 ||   |   |   ||   |   |   ||   |   |   ||   |   |   | CC or IMM ->CC    |   |   |   |   |(6)|

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| PSH(4)    | PSHS  | 34| 5+| 2 ||   |   |   ||   |   |   ||   |   |   ||   |   |   | Push reg.->S stack| . | . | . | . | . |

|           | PSHU  | 36| 5+| 2 ||   |   |   ||   |   |   ||   |   |   ||   |   |   | Push reg.->U stack| . | . | . | . | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| PUL(4)    | PULS  | 35| 5+| 2 ||   |   |   ||   |   |   ||   |   |   ||   |   |   | Pull reg.<-S stack| . | . | . | . | . |

|           | PULU  | 37| 5+| 2 ||   |   |   ||   |   |   ||   |   |   ||   |   |   | Pull reg.<-U stack| . | . | . | . | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| ROL       | ROLA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 49| 2 | 1 | A/---------------\| . | * | * | * | * |

|           | ROLB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 59| 2 | 1 | B\-||<|||||||||<-/| . | * | * | * | * |

|           | ROL   |   |   |   || 09| 6 | 2 || 69| 6+| 2+|| 79| 7 | 3 ||   |   |   | M   c b7 <<< b0   | . | * | * | * | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| ROR       | RORA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 46| 2 | 1 | A/---------------\| . | * | * | . | * |

|           | RORB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 56| 2 | 1 | B\->||>|||||||||-/| . | * | * | . | * |

|           | ROR   |   |   |   || 06| 6 | 2 || 66| 6+| 2+|| 76| 7 | 3 ||   |   |   | M    c b7 >>> b0  | . | * | * | . | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| RTI       |       |   |   |   ||   |   |   ||   |   |   ||   |   |   || 3B|15 | 1 | Return from int.  |   |   |   |   |(6)|

|           |       |   |   |   ||   |   |   ||   |   |   ||   |   |   ||   | 6 |   | (6~ if fast int.) |   |   |   |   |   |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| RTS       |       |   |   |   ||   |   |   ||   |   |   ||   |   |   || 39| 5 | 1 | Return from sub.  | . | . | . | . | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| SBC       | SBCA  | 82| 2 | 2 || 92| 4 | 2 || A2| 4+| 2+|| B2| 5 | 3 ||   |   |   | A -M -C ->A       |(7)| * | * | * | * |

|           | SBCD  | C2| 2 | 2 || D2| 4 | 2 || E2| 4+| 2+|| F2| 5 | 3 ||   |   |   | B -M -C ->B       |(7)| * | * | * | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| SEX       |       |   |   |   ||   |   |   ||   |   |   ||   |   |   || 1D| 2 | 1 | Sign extend B to A| . | * | * | 0 | . |

-----------------------------------------------------------------------------------------------------------------------------

-----------------------------------------------------------------------------------------------------------------------------

|Instruction| Forms | Addressing Modes                                              | Description       | Condition Codes   |

|           |       |---------------------------------------------------------------|                   |-------------------|

|           |       | Immediate || Direct    || Indexed(1)|| Extended  || Inherent  |                   | b5| b3| b2| b1| b0|

|           |       |-----------||-----------||-----------||-----------||-----------|                   |---|---|---|---|---|

|           |       | Op| ~ | # || Op| ~ | # || Op| ~ | # || Op| ~ | # || Op| ~ | # |                   | H | N | Z | V | C |

|===========|=======|===|===|===||===========||===|===|===||===|===|===||===|===|===|===================|===|===|===|===|===|

| ST        | STA   |   |   |   || 97| 4 | 2 || A7| 4+| 2+|| B7| 5 | 3 ||   |   |   | A ->M             | . | * | * | 0 | . |

|           | STB   |   |   |   || D7| 4 | 2 || E7| 4+| 2+|| F7| 5 | 3 ||   |   |   | B ->M             | . | * | * | 0 | . |

|           | STD   |   |   |   || DD| 5 | 2 || ED| 5+| 2+|| FD| 6 | 3 ||   |   |   | D ->M:M+1         | . | * | * | 0 | . |

|           | STS   |   |   |   || 10| 6 | 3 || 10| 6+| 3+|| 10| 7 | 4 ||   |   |   | S ->M:M+1         | . | * | * | 0 | . |

|           |       |   |   |   || DF|   |   || EF|   |   || FF|   |   ||   |   |   |                   |   |   |   |   |   |

|           | STU   |   |   |   || DF| 5 | 2 || EF| 5+| 2+|| FF| 6 | 3 ||   |   |   | U ->M:M+1         | . | * | * | 0 | . |

|           | STX   |   |   |   || 9F| 5 | 2 || AF| 5+| 2+|| BF| 6 | 3 ||   |   |   | X ->M:M+1         | . | * | * | 0 | . |

|           | STY   |   |   |   || 10| 6 | 3 || 10| 6+| 3+|| 10| 7 | 4 ||   |   |   | Y ->M:M+1         | . | * | * | 0 | . |

|           |       |   |   |   || 9F|   |   || AF|   |   || BF|   |   ||   |   |   |                   |   |   |   |   |   |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| SUB       | SUBA  | 80| 2 | 2 || 90| 4 | 2 || A0| 4+| 2+|| B0| 5 | 3 ||   |   |   | A -M ->A          |(7)| * | * | * | * |

|           | SUBB  | C0| 2 | 2 || D0| 4 | 2 || E0| 4+| 2+|| F0| 5 | 3 ||   |   |   | B -M ->B          |(7)| * | * | * | * |

|           | SUBD  | 83| 4 | 3 || 93| 6 | 2 || A3| 6+| 2+|| B3| 7 | 3 ||   |   |   | D -M:M+1 ->D      | . | * | * | * | * |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| SWI(5)    | SWI   |   |   |   ||   |   |   ||   |   |   ||   |   |   || 3F|19 | 1 | Software int. 1   | . | . | . | . | . |

|           | SWI2  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 10|20 | 2 | Software int. 2   | . | . | . | . | . |

|           |       |   |   |   ||   |   |   ||   |   |   ||   |   |   || 3F|   |   |                   |   |   |   |   |   |

|           | SWI3  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 11|20 | 2 | Software int. 3   | . | . | . | . | . |

|           |       |   |   |   ||   |   |   ||   |   |   ||   |   |   || 3F|   |   |                   |   |   |   |   |   |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| SYNC      |       |   |   |   ||   |   |   ||   |   |   ||   |   |   || 13| 4+| 1 |Synchronise to int.| . | . | . | . | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| TFR       | R1->R2| 1F| 6 | 2 ||   |   |   ||   |   |   ||   |   |   ||   |   |   | R1 ->R2(2)        | . | . | . | . | . |

|-----------|-------|---|---|---||---|---|---||---|---|---||---|---|---||---|---|---|-------------------|---|---|---|---|---|

| TST       | TSTA  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 4D| 2 | 1 | Test A            | . | * | * | 0 | . |

|           | TSTB  |   |   |   ||   |   |   ||   |   |   ||   |   |   || 5D| 2 | 1 | Test B            | . | * | * | 0 | . |

|           | TST   |   |   |   || 0D| 6 | 2 || 6D| 6+| 2+|| 7D| 7 | 3 ||   |   |   | Test M            | . | * | * | 0 | . |

-----------------------------------------------------------------------------------------------------------------------------

Legend:

Op  Operation Code (Hexadecimal)    /M  Complement of M                 *   Test and set if true (cleared otherwise)

~   Number of MPU Cycles            ->  Transfer into                   -   Not Affected

#   Number of Probram Bytes         H   Half Carry (from bit 3)         CC  Condition Code Register

+   Arithmetic Plus                 N   Negative (sign bit)             :   Concatenation

-   Arithmetic Minus                Z   Zero Result                     or  Logical OR

IMM Immediate mode data             V   Overflow, 2s Complement         ^   Logical AND

C   Carry from ALU                  xor Logical Exclusive OR

Notes:

(1) This column gives a base cycle and byte count. To obtain total count, add the values obtained from the

INDEXED ADDRESS MODES table.

(2) R1 and R2 may be any pair of 8 bit or any pair of 16 bit registers.

The 8 bit registers are: A, B, CC, DP

The 16 bit registers are: X, Y, U, S, D, PC

(3) EA is the effective address. LEAX ,X+ and LEAX ,X++ do not increment X (same for others).

(4) The PSH and PUL instructions require 5 cycles plus 1 cycle for each byte pushed or pulled.

(5) SWI sets the I and F bits. SWI2 and SWI3 do not affect I and F.

(6) Condition Codes are set as a direct result of the instruction.

(7) Value of the half carry flag is undefined.

(8) Special Case: Carry is the same as B bit 7.

PUSH/PULL POST BYTE                             TRANSFER EXCHANGE POST BYTE

---------------------------------               ---------------------------------

| b7| b6| b5| b4| b3| b2| b1| b0|               | Source        | Destination   |

---------------------------------               ---------------------------------

|   |   |   |   |   |   |   |_____CCR

|   |   |   |   |   |   |_________A           REGISTER FIELD (Source or Destination)

|   |   |   |   |   |_____________B

|   |   |   |   |_________________DPR         0000 =  D (A:B)     0101 =  PC

|   |   |   |_____________________X           0001 =  X           1000 =  A

|   |   |_________________________Y           0010 =  Y           1001 =  B

|   |_____________________________S/U         0011 =  U           1010 =  CCR

|_________________________________PC          0100 =  S           1011 =  DPR

INDEXED ADDRESSING MODES

---------------------------------------------------------------------------------------------------------------------

| Type                      | Forms                 | Non Indirect                  | Indirect                      |

|                           |                       |-------------------------------|-------------------------------|

|                           |                       | Assembler | Postbyte  | + | + | Assembler | Postbyte  | + | + |

|                           |                       | Form      | OP code   | ~ | # | Form      | OP Code   | ~ | # |

|===========================|=======================|===========|===========|===|===|===========|===========|===|===|

| Constant Offset From R    |  No Offset            |  ,R       | 1RR00100  | 0 | 0 |  [,R]     | 1RR10100  | 3 | 0 |

| (twos complement offset)  |  5 Bit Offset         | n,R       | 0RRnnnnn  | 1 | 0 | defaults to 8 bit     |   |   |

|                           |  8 Bit Offset         | n,R       | 1RR01000  | 1 | 1 | [n,R]     | 1RR11000  | 4 | 1 |

|                           | 16 Bit Offset         | n,R       | 1RR01001  | 4 | 2 | [n,R]     | 1RR11001  | 7 | 2 |

|---------------------------|-----------------------|-----------|-----------|---|---|-----------|-----------|---|---|

| Accumulator Offset From R | A -- Register Offset  | A,R       | 1RR00110  | 1 | 0 | [A,R]     | 1RR10110  | 4 | 0 |

| (twos complement offset)  | B -- Register Offset  | B,R       | 1RR00101  | 1 | 0 | [B,R]     | 1RR10101  | 4 | 0 |

|                           | D -- Register Offset  | D,R       | 1RR01011  | 4 | 0 | [D,R]     | 1RR11011  | 7 | 0 |

|---------------------------|-----------------------|-----------|-----------|---|---|-----------|-----------|---|---|

| Auto Increment/Decrement R| Increment By 1        |  ,R+      | 1RR00000  | 2 | 0 |not allowed|           |   |   |

|                           | Increment By 2        |  ,R++     | 1RR00001  | 3 | 0 |  [,R++]   | 1RR10001  | 6 | 0 |

|                           | Decrement By 1        |  ,-R      | 1RR00010  | 2 | 0 |not allowed|           |   |   |

|                           | Decrement By 2        |  ,--R     | 1RR00011  | 3 | 0 |  [,--R]   | 1RR10011  | 6 | 0 |

|---------------------------|-----------------------|-----------|-----------|---|---|-----------|-----------|---|---|

| Constant Offset From PC   |  8 Bit Offset         | n,PCR     | 1XX01100  | 1 | 1 | [n,PCR]   | 1XX11100  | 4 | 1 |

| (twos complement offset)  | 16 Bit Offser         | n,PCR     | 1XX01101  | 5 | 2 | [n,PCR]   | 1XX11101  | 8 | 2 |

|---------------------------|-----------------------|-----------|-----------|---|---|-----------|-----------|---|---|

| Extended Indirect         | 16 Bit Address        |           |           |   |   | [n]       | 10011111  | 5 | 2 |

---------------------------------------------------------------------------------------------------------------------

R = X, Y, U or S        X = 00      Y = 01

X = Don't Care          U = 10      S = 11

RELATIVE BRANCH INSTRUCTIONS

-------------------------------------------------------------   -------------------------------------------------------------

|Instr. | Forms | Addressing| Condition for                 |   |Instr. | Forms | Addressing| Condition for                 |

|       |       | Mode      | Branch                        |   |       |       | Mode      | Branch                        |

|       |       |-----------|                               |   |       |       |-----------|                               |

|       |       | Relative  |                               |   |       |       | Relative  |                               |

|       |       |---|---|---|                               |   |       |       |---|---|---|                               |

|       |       | OP| ~ | # |                               |   |       |       | OP| ~ | # |                               |

|=======|=======|===|===|===|===============================|   |=======|=======|===|===|===|===============================|

| BCC   | BCC   | 24| 3 | 2 | C = 0                         |   | BLS   | BLS   | 23| 3 | 2 | Result lower or same          |

|       | LBCC  | 10| 5 | 4 |                               |   |       | LBLS  | 10| 5 | 4 | (unsigned)                    |

|       |       | 24|(6)|   |                               |   |       |       | 23|(6)|   | C or Z = 1                    |

|-------|-------|---|---|---|-------------------------------|   |-------|-------|---|---|---|-------------------------------|

| BCS   | BCS   | 25| 3 | 2 | C = 1                         |   | BLT   | BLT   | 2D| 3 | 2 | Result < 0                    |

|       | LBCS  | 10| 5 | 4 |                               |   |       | LBLT  | 10| 5 | 4 | (signed)                      |

|       |       | 25|(6)|   |                               |   |       |       | 2D|(6)|   | N xor V = 1                   |

|-------|-------|---|---|---|-------------------------------|   |-------|-------|---|---|---|-------------------------------|

| BEQ   | BEQ   | 27| 3 | 2 | Z = 1                         |   | BMI   | BMI   | 28| 3 | 2 | N = 1                         |

|       | LBEQ  | 10| 5 | 4 |                               |   |       | LBMI  | 10| 5 | 4 |                               |

|       |       | 27|(6)|   |                               |   |       |       | 28|(6)|   |                               |

|-------|-------|---|---|---|-------------------------------|   |-------|-------|---|---|---|-------------------------------|

| BGE   | BGE   | 2C| 3 | 2 | Result >= 0                   |   | BNE   | BNE   | 26| 3 | 2 | Z = 0                         |

|       | LBGE  | 10| 5 | 4 | (signed)                      |   |       | LBNE  | 10| 5 | 4 |                               |

|       |       | 2C|(6)|   | N xor V = 0                   |   |       |       | 26|(6)|   |                               |

|-------|-------|---|---|---|-------------------------------|   |-------|-------|---|---|---|-------------------------------|

| BGT   | BGT   | 2E| 3 | 2 | Result > 0                    |   | BPL   | BPL   | 2A| 3 | 2 | N = 0                         |

|       | LBGT  | 10| 5 | 4 | (signed)                      |   |       | LBPL  | 10| 5 | 4 |                               |

|       |       | 2E|(6)|   | (N xor V) or Z = 0            |   |       |       | 2A|(6)|   |                               |

|-------|-------|---|---|---|-------------------------------|   |-------|-------|---|---|---|-------------------------------|

| BHI   | BHI   | 22| 3 | 2 | Result higher                 |   | BRA   | BRA   | 20| 3 | 2 | Branch always                 |

|       | LBHI  | 10| 5 | 4 | (unsigned)                    |   |       | LBRA  | 16| 5 | 3 |                               |

|       |       | 22|(6)|   | C or Z = 0                    |   |-------|-------|---|---|---|-------------------------------|

|-------|-------|---|---|---|-------------------------------|   | BRN   | BRN   | 21| 3 | 2 | Branch never                  |

| BHS   | BHS   | 24| 3 | 2 | Result higher or same         |   |       | LBRN  | 10| 5 | 4 |                               |

|       | LBHS  | 10| 5 | 4 | (unsigned)                    |   |       |       | 21|(6)|   |                               |

|       |       | 24|(6)|   | C = 0                         |   |-------|-------|---|---|---|-------------------------------|

|-------|-------|---|---|---|-------------------------------|   | BSR   | BSR   | 8D| 7 | 2 | Branch to subroutine          |

| BLE   | BLE   | 2F| 3 | 2 | Result <= 0                   |   |       | LBSR  | 17| 9 | 3 |                               |

|       | LBLE  | 10| 5 | 4 | (signed)                      |   |-------|-------|---|---|---|-------------------------------|

|       |       | 2F|(6)|   | (N xor V) or Z = 1            |   | BVC   | BVC   | 28| 3 | 2 | V = 0                         |

|-------|-------|---|---|---|-------------------------------|   |       | LBVC  | 10| 5 | 4 |                               |

| BLO   | BLO   | 25| 3 | 2 | Result lower                  |   |       |       | 28|(6)|   |                               |

|       | LBLO  | 10| 5 | 4 | (unsigned)                    |   |-------|-------|---|---|---|-------------------------------|

|       |       | 25|(6)|   | C = 1                         |   | BVS   | BVS   | 29| 3 | 2 | V = 1                         |

-------------------------------------------------------------   |       | LBVS  | 10| 5 | 4 |                               |

|       |       | 29|(6)|   |                               |

-------------------------------------------------------------

Notes:

(1) Cycle counts in parenthesis are if Long Branch is taken.

(2) Condition codes are unchanged by the branch instructions.

(3) BXX branches are -128 to +127 bytes from the following

opcode.

(4) LBXX branches cover the entire 64 Kbyte memory range.

---------------------------------   ------------------------------------------

| CCR   | Result (BXX or LBXX)  |   | Test  | Result (BXX or LBXX)           |

| bit   |-----------------------|   |       |--------------------------------|

|       | True (1)  | False (0) |   |       | Signed        || Unsigned      |

|-------|-----------|-----------|   |       |---------------||---------------|

| N (b3)| MI        | PL        |   |       | True  | False || True  | False |

| Z (b2)| EQ        | NE        |   |-------|-------|-------||-------|-------|

| V (b1)| VS        | VC        |   | r > m | GT    | LE    || HI    | LS    |

| C (b0)| CS        | CC        |   | r >=m | GE    | LT    || HS    | LO    |

---------------------------------   | r = m | EQ    | NE    || EQ    | NE    |

| r <=m | LE    | GT    || LS    | HI    |

| r < m | LT    | GE    || LO    | HS    |

------------------------------------------

Note:

(1) r= Register, m= Memory.

REGISTERS                               STACKING ORDER                  INTERRUPT VECTORS

b15           b8 b7            b0       |           |                    ___________    End of Memory ($FFFF)

---------------------------------       |===========|                   | Restart   |

D   | A accumulator | B accumulator |       | PC   Lo   |                   |___________|   $FFFE

|-------------------------------|       |-----------|                   | /NMI      |

X   | index register                |       | PC   Hi   |                   |___________|   $FFFC

|-------------------------------|       |===========|                   | SWI       |

Y   | index register                |       | U/S  Lo   |                   |___________|   $FFFA

|-------------------------------|       |-----------|   Pull            | /IRQ      |

U   | user stack                    |       | U/S  Hi   |   (high memory)   |___________|   $FFF8

|-------------------------------|       |===========|                   | /FIRQ     |

S   | hardware stack                |       | Y    Lo   |   /\              |___________|   $FFF6

|-------------------------------|       |-----------|   ||              | SWI2      |

PC  | program counter               |       | Y    Hi   |   ||              |___________|   $FFF4

---------------------------------       |===========|   ||              | SWI3      |

DP | direct page   |       | X    Lo   |   ||              |___________|   $FFF2

|---------------|       |-----------|   ||              | Reserved  |

CC |E|F|H|I|N|Z|V|C|       | X    Hi   |   \/              |___________|   $FFF0

-----------------       |===========|                   |           |

Entire flag _____| | | | | | | |        | DP        |   Push

FIRQ mask   _______| | | | | | |        |===========|   (low memory)

Half Carry  _________| | | | | |        | B (D Lo)  |

IRQ mask    ___________| | | | |        |-----------|

Negative    _____________| | | |        | A (D Hi)  |

Zero        _______________| | |        |===========|

Overflow    _________________| |        | CC        |

Carry       ___________________|        |===========|

|           |

ASCII CHARACTER SET

------------------------------------------

| Least || Most Significant Character    |

| Sig.  ||-------------------------------|

| Char. || 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |

|=======||===|===|===|===|===|===|===|===|

|   0   ||NUL|DLE| SP| 0 | @ | P | ` | p |  NUL Null                    DLE Data link escape

|-------||---|---|---|---|---|---|---|---|

|   1   ||SOH|DC1| ! | 1 | A | Q | a | q |  SOH Start of heading        DC1 Device control 1 (XON,  Control Q)

|-------||---|---|---|---|---|---|---|---|

|   2   ||STX|DC2| " | 2 | B | R | b | r |  STX Start of text           DC2 Device control 2

|-------||---|---|---|---|---|---|---|---|

|   3   ||ETX|DC3| # | 3 | C | S | c | s |  ETX End of text             DC3 Device control 3 (XOFF, Control S)

|-------||---|---|---|---|---|---|---|---|

|   4   ||EOT|DC4| $ | 4 | D | T | d | t |  EOT End of transmission     DC4 Device control 4

|-------||---|---|---|---|---|---|---|---|

|   5   ||ENQ|NAK| % | 5 | E | U | e | u |  ENQ Enquiry                 NAK Negative acknowledge

|-------||---|---|---|---|---|---|---|---|

|   6   ||ACK|SYN| & | 6 | F | V | f | v |  ACK Acknowledge             SYN Synchronous idle

|-------||---|---|---|---|---|---|---|---|

|   7   ||BEL|ETB| ' | 7 | G | W | g | w |  BEL Bell                    ETB End of transmission block

|-------||---|---|---|---|---|---|---|---|

|   8   || BS|CAN| ( | 8 | H | X | h | x |  BS  Backspace               CAN Cancel

|-------||---|---|---|---|---|---|---|---|

|   9   || HT| EM| ) | 9 | I | Y | i | y |  HT  Horizontal tab          EM  End of medium

|-------||---|---|---|---|---|---|---|---|

|   A   || LF|SUB| * | : | J | Z | j | z |  LF  Line feed               SUB Substitute (Control Z)

|-------||---|---|---|---|---|---|---|---|

|   B   || VT|ESC| + | ; | K | [ | k | { |  VT  Vertical tab            ESC Escape

|-------||---|---|---|---|---|---|---|---|

|   C   || FF| FS| , | < | L | \ | l | | |  FF  Form feed               FS  File separator

|-------||---|---|---|---|---|---|---|---|

|   D   || CR| GS| - | = | M | ] | m | } |  CR  Carriage return         GS  Group separator

|-------||---|---|---|---|---|---|---|---|

|   E   || SO| RS| . | > | N | ^ | n | ~ |  SO  Shift out               RS  Record separator

|-------||---|---|---|---|---|---|---|---|

|   F   || SI| US| / | ? | O | _ | o |DEL|  SI  Shift in                US  Unit separator

------------------------------------------

SP  Space

Note:

(1) Control key used with characters in columns 4                   DEL Delete

and 5 generates the codes in columns 0 and 1.

Revisions:

(1) 8-5-97  F. Wilson:  Removed extra data on PUL instruction.

(2) 4-14-98 F. Wilson:  Added Control Q and S to ASCII set and

changed Note 1.

(3) 6-14-98 F. Wilson:  Interchanged the - and _ characters in

the ASCII CHARACTER SET section.