NOTE: Downloaded from the Motorola "freeware" BBS (512 891-3733)

BULLETIN "asm_problem"                  Mar 16 90

Assembler Phasing errors.

I have received a number of E-mail messages concerning assembler

"Phasing Errors". There seems to be a bit of confusion about what

this message means. Simply, it means that the assembler found a

symbol with a different address on pass #2 than it had on pass

#1. So the two passes are "out of phase".

Most assemblers, including our FREEWARE assemblers, make two

passes at your source code.

1) The first pass is used to build a symbol table which assigns

an address or data value to each of the symbols (labels) used in

your source code. At the end of pass #1, all symbols in your

program must have a value assigned to them.

2) If there were no "fatal" errors in pass #1, then pass #2

begins. Pass #2 generates the actual machine code for your program.

It reads the source code a second time. Each time that a symbol

is found in the source code, the assembler looks into the symbol

table for an address / data value to assign.

3) Both passes keep track of the current program address by

means of a pseudo-PC (program counter). When pass #2 extracts the

address for a symbol from the symbol table, it compares that value

to it's own pseudo-PC value. They had better agree! When they do

not agree. that is a phasing error......

This means that between pass #1 and pass #2 some instruction

has changed it's length (# of bytes). How can this happen? The most

common way relates to addressing modes. Let's assume for a moment

that we are assembling the following MC68HC05 code fragment:

ORG     $100

CAT		LDA     #$F

STA     DOG

PIG	  	LDX     SAM

.

. more code

.

ORG     $20

DOG	  	RMB     2

.

.

Pass #1 creates an entry in the symbol table for "CAT",

assigning it the address $0100. The load-A immediate instruction

requires two bytes so the pseudo-PC is incremented to $0102. When

it processes the second instruction, the assembler encounters the

label "DOG" in the store-A instruction. At this time there is no

entry in the symbol table for "DOG". This is the Foward-reference-

problem and is the reason that there are two passes to the assembly

process. At any rate, the assembler has *NO* knowledge of the

symbol "DOG". Thus, pass #1 assumes extended addressing for the

store-A instruction. This is a THREE byte instruction so the

pseudo-PC is set to $0105. When the label "PIG" is placed in the

symbol table, it is assigned the address $0105.

This is all well and good until the assembler reaches pass #2.

Starting pass #2, the assembler encounters the symbol "CAT". It

checks the symbol table, finding that "CAT" is assigned the value

$0100. This agrees with the pass #2 pseudo-PC, so all is well. The

assembler then assembles the store-A instruction. In the process,

it checks the symbol table for  the value of "DOG". WHOOPS! It

finds that "DOG" is located in page-0. This being the case, the

assembler generates a store-A with DIRECT addressing mode. This

is a TWO byte instruction. Thus, the pseudo-PC is set to $0104.

There is no error message generated yet, but the stage is set.

When pass #2 encounters the symbol "PIG", it get the value $0105

from the symbol table, compares that to it's pseudo-PC which is

$0104, finds that they don't agree, and begins griping at you that

it has a PHASING error.

OK. So what can you do? There are two ways to handle this. First

and best, you should define "DOG" **before** it is encountered in

the program. Secondly, you can "bandaid" it by forcing the direct

addressing mode in the store-A instruction.

This is not the only way to generate a PHASING error, but it's

probably the most common mistake. In all cases, a phasing error

means that pass #1 and pass #2 disagree. Look just ahead of the

first error message (but after the last symbol) for the

villian.........

Sysop....

BULLETIN "asm_problem"                    Mar 16 90

Lost Sectors with AS(n).EXE assemblers.

The AS assemblers for PC type machines (MSDOS and PCDOS) have

a problem which occurs on certain exit conditions. The bug causes

some sectors of your disk to be deallocated (not returned to the

Free List). This does not damage any of your files, but causes your

disk to be fractionally smaller.

The cure is to run DSKCHK on a regular basis. (This is an

EXCELLENT idea regardless). DSKCHK finds the lost sectors and

allows you to recover them......

Sysop.....

Late FLASH:

Due to the generosity of one of our users, we now have a fix for this

problem. The file "CLUSTER.ARC" in the MCU directory contains the source

code which corrects the cluster loss.

Thanks, Bruce!

NOTE: The cluster fix may NOT cure the problem, depending on the C compiler

used.