6809 DEVELOPMENT BOARD:

Frank Wilson

P.O. Box 55

Tomales, CA 94971

(707) 878-2611

fwilson@svn.net

The PC is a very powerful machine with all sorts of neat

programs, great graphics capability and tremendous data storage.

However, I like to tinker with electronics and small computers

are great for that.  There are a lot of commercial development

boards that run from a PC, but I wanted one that was low cost,

easy to modify and with everything in the public domain.  As a

former owner of a Radio Shack Color Computer I got to know the

6809 processor and was impressed by its compact and flexible

instruction set.  It makes ideal "training wheels" for learning

to program in machine language.  The problem is that the Color

Computer is no longer made and never had a strong operating

system similar to CPM.  There were a lot of users, and the spe-

cial interest magazine Rainbow rivaled Byte in size.  When the

Color Computer went out of production everything seemed to col-

lapse.  At least, that was my impression.

My idea was to have the 6809 board communicate with a PC via

a serial RS232 data port.  I had developed a similar interface

for the CoCo, using its' serial printer port.  Software was in

ROM that plugged into the cassette port.  The interface is rela-

tively bombproof and can run at surprisingly high speed (up to

115.2 Kbaud) on a 2 Mhz CPU.  To keep things simple (and just to

see if it could be done) the serial "device" used is a PIA, as in

the CoCo interface.  It senses the incoming signal level and

software is used for decoding.  This is popularly known as a "bit

banger".  When the board is powered up a data loader program in

ROM waits for a machine language program to be downloaded.  A

cross compiler from the Motorola "freeware" bulletin board and a

small editor are used to develop 6809 programs.  To speed up

downloading to the board, the output of the compiler is sent via

COM1 by a data conversion program.  Compiler output is in a human

readable version of hexadecimal representation called "S19"

records.  The conversion program changes it back to binary for-

mat, which is slightly less than half the size of the original

and much more easily decoded.  In fact, programs can be sent at

the 115.2 Kbaud maximum PC rate.  This is as fast as the receiv-

ing software can accept, and for greater operating margin the

loader operates at 57.6 Kbaud.

Using software to generate and receive RS232 serial data

requires special care, taking 100% of the CPUs' time while a

transfer is in progress.  To make things a little easier the

RS232 input to the board goes both to a PIA data input pin and

one of the PIA interrupts.  This way, if the user attempts to

send data to the board while it is outputting something the fact

can be sensed and transfer aborted.  Most of the time the PC acts

as an ACSII terminal, and a small communication program is used

that starts up quickly and handles a simple protocol.  It is

combined with the fast loader, and the result is quick startup.

The system can be up and running in a few seconds, which removes

a big source of frustration.  I have a small machine language

monitor to use in program development, and there is no incentive

to put it in ROM.

The minimal version of the board uses a CPU, PIA, EEPROM,

static RAM, single chip RS232 interface and a couple of addition-

al IC's to handle signal decoding.  I added a second RAM and a

clock divider chip, bringing the memory up to 64 Kbytes and

making a fixed frequency source of interrupts available.  The

interrupts are to experiment with multitasking, and the large

amount of RAM allowed Dieter Flunkerts version of Small-C (from

the C Users Group) to self compile.  The original PC communica-

tion program had to be expanded so that Small-C could use the PC

as a file server, but the combination of fast program downloading

and the simple and powerful 6809 instruction set made everything

reasonable.

The latest project is an emulator for the Digital Equipment

Corporation PDP8.  It was probably the original minicomputer and

a very interesting machine.  Its features were primitive enough

to give a modern programmer the shakes, yet the darn thing ran a

multitasking wafer test system where I used to work.  The earli-

est models came with a Teletype and loaded programs from paper

tape at 10 characters a second.  Our setup was a little more

modern and had a fast paper tape reader and magnetic tape cas-

sette drives.  You still had to talk to it with a Teletype, and

the invention of the CRT based terminal seemed like the greatest

thing since sliced bread.  Our "8" never did graduate from mag-

netic core memory.  One day I used a magnifying glass and, yep,

the memory boards really did have tiny toroids strung on thin

wires.  What did one do for repair, hire a magician?

The EEPROM is 2 Kbytes, with only 1 Kbyte used.  It works in

conjunction with a PIA, which has two sections referred to as A

and B.  The serial data lines are on the A side, so the ROM has

routines to handle access without interference.  Side B has no

restrictions and can directly drive the bases of darlington

transistors at 1 milliampere (minimum) at 1.5 volts.  In practice

this can drive a LED with enough brightness to use as a night

light.  My setup includes a plug in set of 8 LEDs' that are

programmed with a rotating pattern when the board is not doing

anything else.

Using only half the EEPROM leaves the other half free for

updates without disturbing the original program, and allows more

RAM space with the primitive address decoding used.  Besides,

this reduces the tendency toward adding just one more thing.  The

object is to have a simple system while providing a reasonable

set of interface routines.

The PC communication routine does not update the display

very quickly.  It is a simple loop that sends keyboard data out

the serial port and serial input to the display.  The routine

exits to DOS if Escape is entered from the keyboard or Control C

is sent to the display.  When started, if the name of a file

(.S19 extension assumed) is given on the command line it is

converted to binary and sent at 57.6 Kbaud before the

keyboard/display loop is entered.

For convenience the ROM has a routine called CRpause that

allows screen output to be halted at the end of the current line

if any character on the keyboard has been pressed.  It then waits

for another character to continue.  A special test is made for

the Enter key, which allows a software check to abort output.

This is similar to the old Flex operating system, and I find it

much more convenient than using control characters.  One problem

is that reading the PIA clears the input interrupt.  A copy of

the output state is kept in RAM so the serial output routine can

avoid reading the PIA data register.

Since any system as simple as the present one undergoes

modification it seemed necessary to allow for changed ROM subrou-

tine and data locations.  The way I do it is to use a list of

equivalents to the desired locations, replacing them with point-

ers.  Each pointer is guaranteed to have a different value,

reducing potential conflicts.  The list of replacement vectors is

pointed to by a RAM location, allowing a modified set to be

downloaded.  When a program is started the replacement routine is

pointed to by the X index register.  Call the routine with D

pointing to the list.

As a convenience the program loader can also accept "S19"

records, but only at low speed.  Binary data at 57.6 Kbaud is

over 10 times faster than S records at 9600 baud.  An experimen-

tal 115.2 Kbaud loader also worked, but there was no margin for

skew or signal degradation (use a short serial data cable).  I

think this results in almost 25 times speedup, and is about the

limit with a "bit banger".  The loader has settable upper and

lower load limits, and exits when an "S9" record is received.

At times a bit banger can start receiving in the middle of a

data string, so the "ClrLine" subroutine waits till the RS232

input line has been quiet long enough to assure that an ASCII

input character will not be chopped up.

Note that although a PC can send at up to 115.2 Kbaud, there

is a serious problem when it is receiving.  The PC can be (and

usually is) interrupted for internal bookkeeping, such as memory

refresh and clock update.  When this happens a few input charac-

ters are lost if operating at much over 9600 baud.  Arranging for

the PC to echo the received character means the next character

can be sent as soon as the echo starts, resulting in a certain

amount of overlap.

SUBMISSION DISK DIRECTORY:

|

|   LOAD.EXE                  PC program loader and monitor.

|   BOOT09.CRE                ROM access vector equates.

|   BOOTMON.S19               6809 monitor (object code).

|   FLOAT09.S19               Floating point test (object code).

|   AS9.EXE                   6809 cross compiler.

|   6809_DEV.DOC              This file.

|

+---6809CPU              Documentation for the 6809 CPU:

|       6809REF.DOC           6809 reference card (132 column).

|       ASREFNEW.MAN          Cross compiler reference manual.

|       ASEMBLER.DOC          Short cross compiler descriptions.

|       README.DOC            Additional cross compiler data.

|       ASMPROB.DOC           Explanation of a common problem.

|

+---6809ROM              6809 board ROM and monitor files:

|       BOOTMON.ASM           Monitor source code.

|       BOOT09.ASM            Bootup ROM source code.

|       BOOT09.BAT            Compiles ROM.

|       BOOTMON.BAT           Compiles the monitor.

|

+---6809PCB              6809 board construction documentation:

|       6809WIRE.DOC          Wire list.

|       6809SCH.DOC           Schematic.

|       6809LAY.DOC           Parts layout.

|       6809PTS.DOC           Suggested part order form.

|

+---PDP8_EMU             PDP8/E emulator files:

|       PDPASM.EXE            PDP8 cross compiler.

|       PDPASM.C              Cross compiler source.

|       ASM.BAT               Makes the cross compiler.

|       FOCAL69.BIN           PDP8/E scientific calculator.

|       PDP8E.ASM             Emulator source.

|       PDP8E.BAT             Compiles the emulator.

|       PDP8E.S19             Emulator object code.

|       TEST.ASM              PDP8/E instruction test.

|       RUN8E.BAT             Runs a ".BIN" file on the emulator.

|       TEST.BAT              Compiles the test file.

|       PDPLIT.DOC            PDP8 literature sources.

|

+---FLOAT09              Floating point routines:

|       TRIG.ASM              Trig functions.

|       FLOAT09.ASM           Test program.

|       6809FP11.ASM          Floating point.

|       FLOAT09.BAT           Compiles the test program.

|

+---68XXASM              68XX cross compiler source:

|       AS9.BAT               Makes the 6809 cross compiler.

|       OUTPUT.C

|       FFWD.C

|       AS.C

|       AS.H

|       AS0.C

|       AS1.C

|       AS11.C

|       AS4.C

|       AS5.C

|       AS9.C

|       DO0.C

|       DO1.C

|       DO11.C

|       DO4.C

|       DO5.C

|       DO9.C

|       EVAL.C

|       PSEUDO.C

|       SYMTAB.C

|       TABLE0.H

|       TABLE1.H

|       TABLE11.H

|       TABLE4.H

|       TABLE5.H

|       TABLE9.H

|       UTIL.C

|       README.DOC            Explanation of the files.

|

+---SMALL-C              Dieter Flunkerts Small-C:

|   |   CC09.BAT              Compiles C files to 6809 object.

|   |   SEND09.BAT            Loads and starts object code.

|   |   SIEVE.C               Standard sieve test program.

|   |   STEST1.C              Multiple loop sieve program.

|   |   LINK09.BAT            Board resident data link.

|   |

|   +---68LINK           Board resident data link files:

|   |       RELOC.ASM

|   |       LINK.ASM

|   |       LINKDEFS.ASM

|   |       BLKIN.ASM

|   |       EQUATES.ASM

|   |       MEMTEST.ASM

|   |       CMDARGS.ASM

|   |       INIT09.ASM

|   |       TSERIN.ASM

|   |       SERIO.ASM

|   |       TESTMON.ASM

|   |       LINK09.CRE

|   |       TESTMON.BAT       Compiles link (including monitor).

|   |       LINK09.S19        Link object code.

|   |

|   +---COMPILER         Source code for Dieters' Small-C:

|   |   |   CC1.C

|   |   |   CC9.C

|   |   |   CC2.C

|   |   |   CC3.C

|   |   |   CC4.C

|   |   |   CC5.C

|   |   |   CC6.C

|   |   |   CC7.C

|   |   |   CC8.C

|   |   |   CC11.C

|   |   |   CC21.C

|   |   |   CC31.C

|   |   |   CC41.C

|   |   |   CC51.C

|   |   |   CC61.C

|   |   |   CC71.C

|   |   |   CC6809.C

|   |   |   C_COMP09.BAT      Makes Small-C cross compiler.

|   |   |   CC09.EXE

|   |   |   CC.DOC            Small C documentation.

|   |   |   CTEST.BAT         Small-C self compile test.

|   |   |   STDIO_09.BAT      Set up SDTIO.H file.

|   |   |   STDIOHDR.H

|   |   |   STDIOEND.H

|   |   |   CC09.C

|   |   |   CCDEF.C

|   |   |   CC6809O.C

|   |   |   CC.C

|   |   |   C.R09

|   |   |

|   |   \---LIB09        Small C library files:

|   |           STDIO.H

|   |           STDLIB.H

|   |           CTYPE.H

|   |           STRING.H

|   |

|   \---PCLINK           PC resident data link files:

|           LINK09.EXE        Link object.

|           LINK09.C          Link source.

|           LINK09PC.BAT      Compiles the link.

|

\---PCFILES              PC file source:

SAVE.C                Sends data block to PC.

LOAD.C                Loader/monitor source.

PCLOAD.BAT            Compiles the loader/monitor.

DIRECTORY NOTES:

(1)  In the 6809CPU directory, all but the README.DOC file are

from the Motorola public domain BBS at: (512) 891-3733.

(2)  In the FLOAT09 directory the TRIG.ASM and 6809FP11.ASM

files are also from the Motorola BBS.  They originaly

ran on a 68HC11 and have been modified for the 6809.

(3)  Files in the 68XXASM directory are also from the Motorola

BBS, with some modifications and the addition of README.DOC

(4)  Files in the COMPILER and LIB09 directories are based on

Dieter Flunkerts' version of Ron Cains' Small C, and are

from the C Users Group Disk #221.01 on the Walnut Creek

CD-ROM.  Addresses are:

The C Users Group

1601 W. 23rd St. Suite 200

Larwence, KS 66046

Walnut Creek CDROM

1547 Palos Verdes Mall, Suite 260

Walnut Creek, CA 94596

(510) 674-0783

(5)  FOCAL69 in the PDP8_EMU directory is courtesy of the Digital

Equipment Corporation.

EXAMPLES:

---------

(1) Floating point test:

C:\TCJ_SUB.09>load float09

0  A + B

1.  A -B

2.  A X B

3.  A/B

4.  Square root of A

5.  A = Pi

6.  Exchange A and B

7.  Move A to B

8.  Compare A to B

9.  A= Sine(A)

10.  A= Cosine(A)

11.  A= Tangend(A)

12.  A= ArcSine(A)

13.  A= ArcCosine(A)

14.  A= ArcTangent(A)

15.  Load A and B

16.  Exit program

Selection? 2

A X B

Input data for A: 1234

Input data for B: 5678

A=  7.006652E+06

B=  5678.

0  A + B

1.  A -B

2.  A X B

3.  A/B

4.  Square root of A

5.  A = Pi

6.  Exchange A and B

7.  Move A to B

8.  Compare A to B

9.  A= Sine(A)

10.  A= Cosine(A)

11.  A= Tangend(A)

12.  A= ArcSine(A)

13.  A= ArcCosine(A)

14.  A= ArcTangent(A)

15.  Load A and B

16.  Exit program

(2) Small C test:

NOTE: LINK09 is only loaded once. Then the

compile, load/run and exit steps can

be repeated.

C:\TCJ_SUB.09\SMALL-C>link09

==> 6809 machine language monitor <==

(? or / to list options)

> L C:\TCJ_SUB.09\SMALL-C>cc09 sieve

compiling sieve.c

0 error(s) in compilation.

assembling object file

Number of errors 0

C:\TCJ_SUB.09\SMALL-C>send09 sieve

Sending file sieve.S19 to monitor.

File sieve.S19 send, re-entering monitor

> G

Command:

>i<?

> L C:\TCJ_SUB.09\SMALL-C>

(3) Test FOCAL on the PDP8 emulator:

C:\TCJ_SUB.09\PDP8_EMU>run8e focal69

C:\TCJ_SUB.09\PDP8_EMU>..\load pdp8e

PDP8-E EMULATOR

-> Waiting for binary file download at 9600 baud <-

C:\TCJ_SUB.09\PDP8_EMU>copy focal69.bin com1: /b

1 file(s) copied

C:\TCJ_SUB.09\PDP8_EMU>..\load

?=     Print prompt list

K=     Keyboard input

E=     Enable display

D=     Disable display

Space= Next step

C-R=   Exit single stepping

Df= 0 AC=0000 L=0 MQ=0000 U=0 ION=0 NoI=0 Ireq=0000

If= 0 PC=0200 In=5576

*T 1234*5678

= 7006650.0*