Files are for the freeware assemblers

-------------------------------------------

The file ASEMBLER.ARC from the Motorola "freeware" bulletin

board (512 891-3733) contains all the files needed to make

various 8 bit assemblers for the Motorola 8 bit microprocessors,

but some of the files are not the same as the separate versions

from the BBS. Copy ASEMBLER.ARC and "de arc" it. The BBS file

CLUSTER.ARC has some updates and is supposed to fix the lost

cluster problem, but does not. The individual BBS assembler files

FFWD.C, OUTPUT.C and AS.C are the same as those in ASEMBLER.ARC.

Files EVAL.C, PSEUDO.C, SYMTAB.C and UTIL.C are different. EVAL.C

adds right and left shift operators (< and >). PSEUDO.C adds the

SET directive and SYMTAB.C supports the addition. UTIL.C changes

the error printout to use "|" instead of ":". Note that EVAL.C

removes an important part of the cross reference listing. To compile

with MIX-C add definitions for TRUE and FALSE to the file AS.H.

The files in this directory have been updated to assemble with the

MIX-C compiler and keep the full cross reference capability. If the

resulting compiler is stopped before finishing a compilation it will

leave a temporary work file in the directory. Bruce Olney has an

updated version of the 6809 compiler on the BBS that fixes all the

problems.

NEW:

(1) Binary file option (using "b") in addition to "S" record capability.

Direct translation, with "S1" and "S9" changed to $B1 and $B9, and

with a ".B19" file extension in place of ".S19".

(2) Starting address capability. Put origin after END pseudo op in

file, else defaults to start at $0000.

(3) The 6809 assembler checks for dual cycle instructions (certain

long branch instructions) and prints the proper cycles "[m/n]"

where "m" and "n" are the branch not taken and branch taken

cycles.

(4) "EXT" option. Follow with list of labels and they will be printed

out (as equates) if "e" is used in the command line.